T E C H N I C A L R E F E R E N C E Maritime Calculations Reference Engineering Reference Manual — Compliance, Voyage & Charter Performance Prepared for: Poseidon Ledger Sources: IMO · EU Regulations · BIMCO/NYPE · ISO · SNAME/ITTC 23 calculation topics with formulas, variables, worked examples Reference date: July 2026 Poseidon Ledger Engineering Reference Series

How to Use This Reference

This manual is an engineering reference for the principal quantitative methods used in maritime compliance, voyage performance and charter-party evaluation. Each of the twenty-three topics follows the same six-part structure: Formula, Variable Definitions, Units, Worked Example, Practical Use, Limitations and Official Source.

Formulas are drawn from the cited official sources: IMO resolutions (MEPC series) and MARPOL Annex VI, EU regulations (2015/757, 2023/1805, 2003/87/EC as amended by 2023/959), BIMCO and NYPE standard charter forms, and recognised maritime engineering references (ISO 15016, ISO 3046, SNAME, ITTC). Where a parameter (such as a ship-type coefficient) is defined in an annex table that is periodically revised, the structure of the formula is given and the reader is directed to the current source for the numerical value.

Worked examples are illustrative: the numbers are realistic but are chosen to demonstrate the arithmetic, not to assert any specific ship's performance or a particular regulatory outcome. Operators should always compute against the current official text and the ship's own certified data.

Notation follows maritime engineering convention: superscripts are written with the caret (^) symbol and subscripts with the underscore (_) inside formulas, so that expressions read unambiguously in plain text. Units are stated in SI and maritime units (nautical mile, knot, tonne) throughout.

Table of Contents

1. CII — Carbon Intensity Indicator	2

2. AER — Annual Efficiency Ratio	6

3. EEOI — Energy Efficiency Operational Indicator	9

4. EEXI — Energy Efficiency Existing Ship Index	12

5. EEDI — Energy Efficiency Design Index (new ships)	16

6. FuelEU Maritime — Well-to-Wake Intensity & Compliance Balance	19

7. EU ETS — Maritime Emissions & Allowance Surrender	23

8. EU MRV — Monitoring, Reporting, Verification Calculations	27

9. IMO DCS — Data Collection System	30

10. CO2 Emissions from Different Fuel Types	33

11. Fuel Consumption Estimation	36

12. Voyage Distance Calculation	39

13. Great Circle Distance (Haversine / Spherical Law of Cosines)	42

14. ETA Estimation	46

15. Voyage Performance Calculations	49

16. Speed vs Fuel Consumption (Cubic Law & Admiralty)	52

17. Weather Correction Concepts	55

18. Engine Load Calculations	59

19. SFOC — Specific Fuel Oil Consumption	62

20. Carbon Intensity Metrics — Consolidated Comparison	65

21. Port Stay Calculations	68

22. Off-Hire Calculations	71

23. Charter Performance Metrics	75

Note: This Table of Contents is generated via field codes. To ensure page-number accuracy after editing, please right-click the TOC and select "Update Field."

# 1. CII — Carbon Intensity Indicator

The Attained CII expresses a ship's annual operational carbon intensity. It was introduced by IMO MARPOL Annex VI amendments (resolution MEPC.328(70) adopted 2016, applicable from 2023) and refined by MEPC.335(76) and MEPC.336(76). Capacity basis, correction factors and rating bands are set out in Appendix II of MARPOL Annex VI and the associated guidelines.

Formula

The Attained CII is the annual mass of CO2 emitted per unit of transport work:

Attained CII = ( FC_ship x CF ) / ( Capacity x D x fi )

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| FC_ship | Mass of fuel consumed per year, summed over all fuel types | t | from IMO DCS data set |
| CF | Fuel-specific CO2 conversion factor (non-dimensional mass ratio) | t-CO2 / t-fuel | see Topic 11; multiply each fuel by its own CF then sum |
| Capacity | Deadweight (DWT) for most ship types; GT for cruise/ro-ro/passenger | t (or GT) | per Appendix II of Annex VI |
| D | Distance travelled in the reporting year | nm | from noon-report or automatic recording |
| fi | Ship-type capacity correction factor for ice-class, etc. | — | 1.0 where not applicable |

Units

Result is expressed in gCO2/dwt-nm (or gCO2/GT-nm). To convert a t-CO2/t-fuel figure to grams, multiply by 10^6; the resulting CII is reported in gCO2 per dwt-nautical mile. The attained value is compared with the Required CII (a year-specific limit derived from the reference line and an annual reduction factor) to assign a rating A–E.

Worked Example

Illustrative example — a 50,000 DWT bulk carrier consuming 4,800 t of VLSFO (CF = 3.114) and sailing 58,000 nm in the year (fi = 1.0):

Numerator  = FC x CF = 4,800 x 3.114 = 14,947.2 t-CO2

Denominator = Capacity x D = 50,000 x 58,000 = 2.9 x 10^9  dwt-nm

Attained CII = (14,947.2 x 10^6 g) / (2.9 x 10^9 dwt-nm)

= 5.15 gCO2/dwt-nm

Required CII (illustrative) = (year limit) e.g. 4.20 gCO2/dwt-nm

Attained > Required  =>  rating in lower band; corrective plan (SEEMP Part III) triggered.

Practical Use

The Attained CII is reported annually in the IMO DCS data set and converted to an A–E rating against the Required CII. It is the IMO's principal operational-efficiency metric for the existing fleet.

- Annual regulatory reporting (MARPOL Annex VI, App. II).

- Charter-party performance discussions (speed/consumption vs rating impact).

- Internal benchmarking of fleet carbon intensity.

Limitations

- It is an annual average; a single bad voyage or ballast legs can distort it.

- Capacity is fixed, so lightly-loaded or ballast-intensive trades appear worse.

- Correction factors and rating band edges are revised by the IMO; always use the current Appendix II values.

- It does not capture well-to-tank emissions (use FuelEU for WtW).

Official Source

- IMO Resolution MEPC.328(70) — MARPOL Annex VI amendments (CII & EEXI).

- IMO Resolution MEPC.335(76) & MEPC.336(76) — CII guidelines and rating.

- MARPOL Annex VI, Appendix II — Reference lines and capacity basis.

- IMO MEPC.1/Circ.684 — Ship Energy Efficiency Regulations.

# 2. AER — Annual Efficiency Ratio

AER is the Sea Cargo Charter's recommended annual intensity metric and is widely used in voluntary and financé-climate reporting. It is structurally close to CII but normalises annual distance by the annualised design speed, so a full year of intensity is estimated even when actual distance is incomplete.

Formula

AER is defined as annual transport-work intensity:

AER = ( MassCO2 ) / ( DWT x D_annualised )

D_annualised = V_design x 24 x days_under_charter

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| MassCO2 | Annual CO2 emitted (all fuel x CF, summed) | t-CO2 | same numerator as CII |
| DWT | Deadweight (capacity for non-passenger ships) | t | GT for cruise/ro-ro passenger ships |
| D_annualised | Annualised distance = design speed x hours under charter | nm | annualises partial-year data |
| V_design | Design (service) speed of the ship | kn | from capacity / trial data |
| days_under_charter | Days the ship was on charter (trading) in the year | d | idle/off-hire days excluded |

Units

Result reported in gCO2/dwt-nm (matching CII units) so AER and CII can be compared directly.

Worked Example

Illustrative — 50,000 DWT container ship, 14,200 t-CO2/year, design speed 22 kn, 320 trading days:

D_annualised = 22 x 24 x 320 = 168,960 nm

AER = (14,200 x 10^6 g) / (50,000 x 168,960)

= 1.68 gCO2/dwt-nm

Practical Use

AER is the metric preferred by the Sea Cargo Charter, Poseidon Principles and many cargo-employer scorecards because it removes the volatility of a single year's distance.

- Voluntary climate-finance disclosure (Poseidon Principles).

- Cargo-owner (charterer) carbon-accountability reporting.

- Multi-year trend analysis where actual distance varies.

Limitations

- Not a regulatory metric; IMO CII is mandatory, AER is voluntary.

- Design-speed annualisation smooths but can mask operational inefficiency.

- Sensitive to the chosen design speed and trading-day definition.

Official Source

- Sea Cargo Charter — Reporting Guidance (Terms & Definitions).

- Poseidon Principles — Technical Guidance (AER methodology).

- IMO MEPC.1/Circ.684 — underlying EEOI/CII methodology.

# 3. EEOI — Energy Efficiency Operational Indicator

The EEOI, defined in IMO MEPC.1/Circ.684, is the operational counterpart to EEDI. It measures actual gCO2 per tonne-nm of transport work over a voyage or period and is the recommended voluntary tool for monitoring operational efficiency.

Formula

The core EEOI is CO2 emitted divided by transport work performed:

j i EEOI = Sum Sum ( FC_ij x CF_i ) j=1 i=1 --------------------------------- j Sum ( m_cargo,j x D_j ) j=1

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| FC_ij | Mass of fuel type i consumed on voyage j | t | per fuel per voyage |
| CF_i | CO2 factor for fuel i | t-CO2/t-fuel | IMO default values, Topic 11 |
| m_cargo,j | Cargo carried (mass or units) on voyage j | t (TEU/pax) | use consistent basis |
| D_j | Distance of voyage j | nm | laden distance where applicable |
| j | Voyage index (1..n) | — | period or voyage basis |

Units

Result in gCO2/t-nm (or gCO2/TEU-nm, gCO2/pax-nm for unitised/passenger trades). A Rolling EEOI over a period averages voyages.

Worked Example

Illustrative — single laden voyage: 1,800 t HFO (CF 3.114), 45,000 t cargo, 4,200 nm:

Numerator = 1,800 x 3.114 = 5,605.2 t-CO2

Denominator = 45,000 x 4,200 = 1.89 x 10^8 t-nm

EEOI = (5,605.2 x 10^6 g) / (1.89 x 10^8 t-nm)

= 29.66 gCO2/t-nm

Practical Use

EEOI is the IMO's recommended voluntary tool for tracking operational efficiency and is often used inside SEEMP Part I and in charter-party performance review.

- SEEMP monitoring (operational efficiency over time).

- Pre-voyage estimation and post-voyage performance review.

- Benchmarking similar ships in a fleet.

Limitations

- Voluntary; not itself a compliance metric (CII is mandatory).

- Ballast legs: per the guidelines they may be excluded or zero-cargo weighted, which changes results materially.

- Cargo-mass basis must be consistent; mixed metric (t vs TEU) is invalid.

Official Source

- IMO MEPC.1/Circ.684 — Guidelines for voluntary use of the EEOI.

- IMO MEPC.1/Circ.684, Appendix — worked examples and rolling average.

# 4. EEXI — Energy Efficiency Existing Ship Index

The EEXI is a one-off technical (design) efficiency index for existing ships ≥ 400 GT, applicable from 2023 under MARPOL Annex VI amendments (MEPC.328(70) and guidelines MEPC.333(76)/MEPC.350(78)). It is structurally identical to EEDI but applied to the existing fleet, often after engine power limitation (EPL/ShaPoLi).

Formula

The Attained EEXI sums the CO2 from main and auxiliary engines and innovative technologies, divided by ship capacity and reference speed:

( P_ME x SFC_ME x CF_ME ) + ( P_AE x SFC_AE x CF_AE ) Attained EEXI = ---------------------------------------------------------------- x f_i x f_w x f_eff + (PTI term) fi x Capacity x V_ref x f_V

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| P_ME | 75% of the (limited) main-engine MCR | kW | after EPL/ShaPoLi if applied |
| SFC_ME | Certified specific fuel consumption of ME at 75% MCR | g/kWh | from NOx Technical File |
| CF_ME | CO2 factor of the ME fuel | t-CO2/t-fuel | Topic 11 |
| P_AE | Auxiliary power at 75% ME load | kW | per EEDI/EEXI formula; simplified for certain ship types |
| Capacity | DWT for most types; GT for passenger/cruise | t / GT | per Annex VI |
| V_ref | Reference speed at 75% MCR (calm water) | kn | from speed-power trial or tank test |
| f_w | Weather/calm-water factor | — | per guidelines |
| f_i | Capacity-correction factor for ice class, etc. | — | 1.0 if not applicable |
| f_eff | Innovative energy-efficiency factor | — | for approved waste-heat/wind devices |

Units

Result in gCO2/t-nm. The Attained EEXI must be <= Required EEXI = (1 - reduction) x Reference line value, where the reference line is (a x Capacity^-c) with ship-type coefficients a, c and a reduction factor specified in the guidelines.

Worked Example

Illustrative structure (a 50,000 DWT bulk carrier, P_ME = 6,000 kW, SFC_ME = 170 g/kWh, HFO CF = 3.114, V_ref = 14.5 kn, P_AE = 350 kW, SFC_AE = 215 g/kWh, all f = 1.0):

ME term  = 6,000 x 170 x 3.114 = 3,176,280

AE term  = 350 x 215 x 3.114   =   234,330

Numerator = 3,176,280 + 234,330 = 3,410,610

Denominator = 50,000 x 14.5 = 725,000

Attained EEXI = 3,410,610 / 725,000 = 4.70 gCO2/t-nm

Required EEXI = (1 - reduction) x Reference line value  ->  compare and, if needed, apply EPL/ShaPoLi and recompute.

Practical Use

EEXI is a regulatory certification (IEE Certificate). It is a one-off but must be re-validated on technical change.

- Compliance certification (International Energy Efficiency Certificate).

- Determining the required engine power limitation (EPL/ShaPoLi).

- Technical benchmark for retrofit decisions.

Limitations

- It is a design (not operational) metric — a compliant EEXI does not mean good CII.

- Reference speed depends on trial/tank data quality; out-of-date data distorts results.

- Ship-type coefficients (a, c) and reduction factors must be taken from the current guidelines.

Official Source

- IMO Resolution MEPC.328(70) — Annex VI amendments introducing EEXI.

- IMO Resolution MEPC.333(76) — EEXI calculation guidelines.

- IMO Resolution MEPC.350(78) — EEXI survey & certification guidelines.

# 5. EEDI — Energy Efficiency Design Index (new ships)

EEDI, mandated by MEPC.203(62) and detailed in MEPC.245(66), is the new-build design efficiency index. It requires each new ship to achieve an Attained EEDI below a phase-specific Required EEDI derived from a ship-type reference line.

Formula

The Attained EEDI for a conventional ship:

( P_ME x SFC_ME x CF_ME ) + ( P_AE x SFC_AE x CF_AE ) + ( P_eff x SFC_eff x CF_eff ) Attained EEDI = ------------------------------------------------------------------------------------------------------------------------- fi x Capacity x V_ref

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| P_ME | 75% of MCR of the main engine | kW | summed if multiple ME |
| SFC_ME | Certified specific fuel consumption of ME | g/kWh | from NOx Technical File |
| CF_ME | CO2 factor of ME fuel | t-CO2/t-fuel | Topic 11 |
| P_AE | Auxiliary engine power required at normal sea load | kW | per EEDI formula |
| P_eff | Power output of innovative electrical/mechanical efficiency tech | kW | e.g. WHR, wind assist |
| Capacity | DWT for most types (GT for cruise/ro-ro passenger) | t / GT | Annex VI definition |
| V_ref | Ship speed at calm water, 75% MCR, design draft | kn | trial or tank-test derived |
| fi, f_w, f_eff | Correction factors | — | ice class, weather, innovative tech |

Units

Result in gCO2/t-nm. Required EEDI = (1 - X/100) x Reference line value, where Reference line = a x Capacity^(-c) (ship-type coefficients from MEPC.245(66)) and X is the phase reduction (Phase 0/1/2/3 — progressively tighter).

Worked Example

Illustrative — new 180,000 DWT bulk carrier, P_ME = 13,000 kW, SFC_ME 168 g/kWh, HFO CF 3.114, V_ref 15.0 kn, simplified:

Numerator approx = 13,000 x 168 x 3.114 = 6,805,000 (ME only, ignoring AE/eff)

Denominator = 180,000 x 15.0 = 2,700,000

Attained EEDI approx = 6,805,000 / 2,700,000 = 2.52 gCO2/t-nm

Required EEDI = (1 - X/100) x (a x Capacity^-c)   ->   compare per current phase.

Practical Use

EEDI is the regulatory requirement that makes new ships progressively more efficient through Phases 0–3 (and onward).

- Mandatory new-build compliance (IEE Certificate).

- Shipyard contract specification & verification trials.

- Setting the baseline against which EEXI measures existing ships.

Limitations

- It captures design, not operations; an EEDI-compliant ship may have a poor CII in service.

- Coefficients a, c and phase reductions are revised periodically.

- Trial/tank-test quality limits reference-speed accuracy.

Official Source

- IMO Resolution MEPC.203(62) — original EEDI amendments.

- IMO Resolution MEPC.245(66) — EEDI calculation guidelines & reference lines.

- IMO Resolution MEPC.251(66) — amendments to EEDI requirements (Phases).

# 6. FuelEU Maritime — Well-to-Wake Intensity & Compliance Balance

FuelEU Maritime (Regulation (EU) 2023/1805, applicable 1 Jan 2025) limits the well-to-wake GHG intensity of energy used on board ships ≥ 5,000 GT calling at EU/EEA ports. It covers CO2, CH4 and N2O, expressed in CO2-equivalent per MJ.

Formula

The annual achieved WtW intensity is a fuel-mass-weighted average of well-to-tank plus tank-to-wake emission factors:

GHG_intensity = Sum_w ( E_w x ( WtT_w + TtW_w ) ) / Sum_w ( E_w )

E_w = m_fuel,w x LCV_w

Compliance balance = ( GHG_intensity - Limit_year ) x Energy_total

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| E_w | Energy content of fuel type w used | MJ | = mass x lower calorific value |
| WtT_w | Well-to-tank emission factor of fuel w (upstream) | gCO2eq/MJ | Annex II of Reg. 2023/1805 |
| TtW_w | Tank-to-wake emission factor of fuel w (combustion) | gCO2eq/MJ | CO2 + CH4 + N2O, in CO2eq |
| m_fuel,w | Mass of fuel w consumed (EU-allocated share) | t | 100% intra-EU; 50% extra-EU |
| LCV_w | Lower calorific value of fuel w | MJ/t | per Annex II |
| Limit_year | Year-specific intensity limit | gCO2eq/MJ | 2% cut in 2025 vs 2020 ref 91.16; 6% in 2030 |

Units

Result in gCO2eq/MJ (CO2-equivalent, using GWP-100: CH4 ~28, N2O ~265). A negative compliance balance = non-compliance; penalty ~€2,400/t VLSFO-equivalent of excess energy, escalating for repeat non-compliance.

Worked Example

Illustrative — a ship uses 5,000 t LNG in the EU-allocated share. Take LCV_LNG ~ 48,000 MJ/t; illustrative combined WtW factor (WtT + TtW) = 18.5 gCO2eq/MJ (values must be confirmed against current Annex II):

E_LNG = 5,000 x 48,000 = 2.4 x 10^8 MJ

GHG_intensity = (2.4e8 x 18.5) / 2.4e8 = 18.5 gCO2eq/MJ  (single fuel)

Limit_2025 = 91.16 x (1 - 0.02) = 89.34 gCO2eq/MJ

Compliance balance = (18.5 - 89.34) x 2.4e8 = negative -> the ship is compliant.

Had intensity been 92.0 gCO2eq/MJ:

Excess = (92.0 - 89.34) x 2.4e8 = 6.38 x 10^7 gCO2eq -> convert to VLSFO-eq energy -> penalty applies.

Practical Use

FuelEU forces the energy/fuel mix toward low-carbon pathways (RFNBOs, biofuels, e-fuels, LNG with methane slip control).

- Annual FuelEU reporting (verified report by 30 June of the following year).

- Fuel-procurement and dual-fuel investment decisions.

- Reward/penalty modelling for fuel-switching scenarios.

Limitations

- Methane slip from LNG engines materially affects the TtW CH4 term and must be engine-specific.

- RFNBO/biofuel emission factors require sustainability certificates to be claimable.

- Limits and penalty rates are updated by the Commission — verify current values.

Official Source

- Regulation (EU) 2023/1805 — FuelEU Maritime (Articles & Annexes I–II).

- European Commission — FuelEU Maritime implementing & delegated acts.

# 7. EU ETS — Maritime Emissions & Allowance Surrender

Shipping entered the EU ETS (Directive 2003/87/EC as amended by (EU) 2023/959) from 2024. Verified CO2 from EU MRV is converted into an allowance-surrender obligation using a phased coverage percentage and geographic allocation.

Formula

The surrender obligation combines the verified emissions with phase-in and geography:

Obligation_t = E_verified x PhasePct_t

E_verified = Sum_voyage ( FC_voy x CF x Geo_voy )

Geo_voy = 100% (intra-EU + at berth) ¦ 50% (extra-EU inbound/outbound)

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| E_verified | Verified CO2 emissions from the EU MRV annual report | t-CO2 | CO2 only 2024-25; CH4+N2O from 2026 |
| PhasePct_t | Phase-in share for reporting year t | % | 40% (2024) / 70% (2025) / 100% (2026+) |
| FC_voy | Fuel consumed on a voyage | t | from MRV monitoring |
| CF | Fuel CO2 conversion factor | t-CO2/t-fuel | Topic 11 |
| Geo_voy | Geographic allocation factor | % | 100% intra-EU & berth; 50% extra-EU legs |

Units

Result in t-CO2 (the surrender obligation). Multiply by the EUA price (€/t-CO2) for the cost. Surrender year = reporting year + 1.

Worked Example

Illustrative — a ship's verified 2024 EU MRV CO2 = 25,000 t, split 16,000 t intra-EU/at-berth and 9,000 t on 50%-legs. PhasePct for 2024 reporting = 40%:

E_verified = 16,000 + 9,000 = 25,000 t-CO2  (already geography-weighted in MRV)

Obligation_2024 = 25,000 x 0.40 = 10,000 EUAs to surrender in 2025

At EUA €70/t: cost approx = 10,000 x 70 = €700,000

For 2025 reporting (70%): Obligation = 25,000 x 0.70 = 17,500 EUAs (surrender 2026).

Practical Use

Determines the annual EUA procurement volume and budget, and feeds charter-party ETS-cost allocation.

- Annual EUA surrender (by September of the following year).

- Charter-party commercial allocation of ETS cost (BIMCO ETS clauses).

- Risk management / hedging of EUA exposure.

Limitations

- Coverage percentage steps each year (40 -> 70 -> 100); multi-year modelling must reflect this.

- CH4 and N2O enter scope from the 2026 reporting year.

- EUA price is volatile; cost is scenario-dependent, not a fixed number.

Official Source

- Directive 2003/87/EC — EU ETS, as amended by Directive (EU) 2023/959 (maritime inclusion).

- Regulation (EU) 2015/757 — EU MRV (provides verified emissions input).

- BIMCO Emissions Trading System Allowance Clause.

# 8. EU MRV — Monitoring, Reporting, Verification Calculations

EU MRV (Regulation (EU) 2015/757, amended by (EU) 2023/2296) requires ships ≥ 5,000 GT to monitor, report and have verified CO2 emissions, fuel consumption, distance, time at sea and transport work for EU/EEA-related activity. The 2023 amendment added CH4 and N2O and expanded scope to additional ship types and sizes in stages.

Formula

Per-voyage emissions, then annual totals and transport work:

E_voy = FC_voy x CF Transport work = Cargo x Distance Intensity = Emissions / Transport work

Annual total = Sum_voyage ( E_voy x Geo_voy ) Geo_voy: 1.0 intra-EU/at berth; 0.5 extra-EU legs

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| FC_voy | Fuel consumed on a voyage (per fuel type) | t | BDN, MFM or tank sounding |
| CF | Fuel CO2 conversion factor | t-CO2/t-fuel | Topic 11; CH4/N2O factors added by 2023 amendment |
| Cargo | Cargo carried on the voyage | t (TEU/pax) | consistent basis per ship type |
| Distance | Voyage distance | nm | port-to-port |
| Geo_voy | Geographic weighting | — | 1.0 or 0.5 |

Units

Emissions in t-CO2 (and t-CH4, t-N2O from the expanded scope). Intensity in gCO2/t-nm (or per TEU/pax-nm). Annual report submitted to THETIS-MRV by 30 April following the reporting year.

Worked Example

Illustrative — voyage Hamburg -> Rotterdam (intra-EU), 350 t VLSFO (CF 3.114), 45,000 t cargo, 320 nm:

E_voy = 350 x 3.114 = 1,089.9 t-CO2  (Geo = 1.0, intra-EU)

Transport work = 45,000 x 320 = 14,400,000 t-nm

Voyage intensity = (1,089.9 x 10^6 g) / 14.4 x 10^6 t-nm = 75.7 gCO2/t-nm

Practical Use

MRV is the verified emissions backbone feeding EU ETS surrender and FuelEU compliance.

- Annual Emissions Report (AER) submitted to THETIS-MRV (by 30 April).

- Source data for EU ETS and FuelEU.

- Public transparency database (THETIS-MRV).

Limitations

- Intra- vs extra-EU classification must be exact; errors corrupt the 1.0/0.5 weighting.

- Cargo recorded at the wrong point distorts transport work.

- Post-2023 amendments add gases and scope — keep Monitoring Plans current.

Official Source

- Regulation (EU) 2015/757 — EU MRV (as amended).

- Regulation (EU) 2023/2296 — MRV amendments (CH4/N2O, scope expansion).

- European Commission Delegated/Implementing Acts on MRV methods.

# 9. IMO DCS — Data Collection System

IMO DCS (MARPOL Annex VI, regulation 22, effective 2018; guidelines MEPC.278(70)) requires ships ≥ 5,000 GT to record and report annual fuel consumption, distance, hours underway and method of data collection. DCS is the data source for CII.

Formula

DCS is primarily data aggregation, with CO2 derived for downstream use:

Annual fuel consumption = Sum_voyage ( FC_voy ) [per fuel type] Derived CO2 = Sum_fuel ( FC_f x CF_f ) Annual distance = Sum_voyage ( D_voy ) Hours underway = Sum_voyage ( t_underway,voy )

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| FC_voy | Fuel consumed per voyage, per fuel type | t | BDN / MFM / sounding |
| CF_f | CO2 factor of fuel f | t-CO2/t-fuel | Topic 11 |
| D_voy | Distance of each voyage | nm | noon reports or automatic |
| t_underway,voy | Underway hours per voyage | h | for average-speed cross-check |

Units

Fuel in metric tonnes, distance in nautical miles, time in hours. The data set is submitted to the Administration/RO for verification; a Statement of Compliance is issued for the next year.

Worked Example

Illustrative — annual totals: FC_HFO = 2,200 t, FC_MGO = 900 t; D = 51,000 nm; hours underway = 3,800 h:

Derived CO2 = 2,200 x 3.114 + 900 x 3.206 = 6,850.8 + 2,885.4 = 9,736.2 t-CO2

Average speed check = 51,000 / 3,800 = 13.4 kn  (cross-check with logbook)

Practical Use

DCS underpins CII rating and the IMO's GHG progress tracking.

- Annual submission to Administration/RO (Statement of Compliance).

- Input to Attained CII calculation (Topic 1).

- IMO fleet-level GHG inventory.

Limitations

- It is a reporting mechanism; accuracy depends on the underlying data quality.

- Method (BDN/MFM/sounding) must match the SEEMP Part I.

- Calendar-bound; partial-year voyages must be allocated correctly.

Official Source

- MARPOL Annex VI, regulation 22 & Appendix IX — DCS data set.

- IMO Resolution MEPC.278(70) — DCS guidelines.

# 10. CO2 Emissions from Different Fuel Types

Tank-to-wake CO2 emissions are computed as the product of fuel mass consumed and the fuel-specific carbon factor CF, specified in MARPOL Annex VI and the IMO Guidelines. CF is a non-dimensional mass ratio (t-CO2 per t-fuel).

Formula

For each fuel consumed:

CO2_i = m_i x CF_i CO2_total = Sum_fuel ( m_i x CF_i )

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| m_i | Mass of fuel type i consumed | t | from BDN/MFM/sounding |
| CF_i | Carbon factor of fuel i | t-CO2/t-fuel | IMO default values, see table below |

Units

Result in t-CO2. Default IMO CF values (MARPOL Annex VI, MEPC.1/Circ.684 and updates):

Worked Example

Illustrative — a voyage burning 1,200 t HFO and 180 t MGO:

CO2_HFO = 1,200 x 3.114 = 3,736.8 t-CO2

CO2_MGO = 180 x 3.206  =   577.1 t-CO2

CO2_total = 3,736.8 + 577.1 = 4,313.9 t-CO2

Practical Use

Universal building block for every CO2-based metric (DCS, MRV, ETS, EEOI, CII, EEXI, EEDI).

- Per-fuel CO2 accounting across all instruments.

- Fuel-switch emissions modelling (HFO -> LNG, biofuel blends).

- Bunker-procurement emissions budgeting.

Limitations

- CF values are tank-to-wake only; they exclude upstream (well-to-tank) emissions.

- Biofuel blends may use lower CF only where sustainability evidence supports it.

- LNG CF does not reflect methane slip — FuelEU WtW factors must be used for that.

Official Source

- MARPOL Annex VI — fuel carbon factors (CF).

- IMO Resolution MEPC.245(66) — EEDI guidelines (CF values).

- IMO MEPC.1/Circ.684 — EEOI guidelines.

# 11. Fuel Consumption Estimation

Fuel consumption is estimated from engine power, load factor and specific fuel consumption. For a multi-engine ship, consumption is summed across main and auxiliary engines. This is the basis of pre-voyage estimation and consumption guarantees.

Formula

Per-engine and total daily consumption:

FC_engine = P_output x SFC x t FC_daily = Sum_engines ( P_i x Load_i x SFC_i ) x 24

SFC = fuel mass / energy output

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| P_output | Engine power output (shaft/electrical) | kW | = MCR x load factor |
| SFC | Specific fuel consumption | g/kWh | from engine shop test / NOx file |
| t | Operating time | h | daily = 24 |
| Load_i | Load factor of engine i (fraction of MCR) | — | 0-1 |
| MCR | Maximum continuous rating | kW | nameplate / limited value |

Units

Fuel mass in g (then converted to t / day). Convert to volume using density (e.g. ~0.99 t/m3 HFO, ~0.86 t/m3 MGO). SFC is typically ~160-180 g/kWh for slow-speed diesels and ~200-220 g/kWh for auxiliaries at service load.

Worked Example

Illustrative — main engine 12,000 kW at 70% load, SFC 170 g/kWh; auxiliary 800 kW at 65% load, SFC 215 g/kWh:

ME daily = 12,000 x 0.70 x 170 x 24 / 10^6 = 34.27 t/day

AE daily = 800 x 0.65 x 215 x 24 / 10^6 =  2.68 t/day

Total daily = 34.27 + 2.68 = 36.95 t/day

Practical Use

Pre-voyage fuel planning, charter-party consumption guarantees, and emissions estimation.

- Pre-voyage bunker requirement and reserve planning.

- Speed/consumption (charter-party) guarantees.

- Inputs to EEOI / CII projection.

Limitations

- SFC varies with load, hull/propeller condition and ambient conditions — a single value is approximate.

- Does not include boiler consumption or emergency-generator use unless separately added.

- Calculated vs measured consumption can diverge 5-10%.

Official Source

- Engine manufacturer shop-test data (SFC curves).

- IMO MEPC.1/Circ.684 — EEOI fuel-accounting methodology.

- SNAME / ITTC marine engineering practice.

# 12. Voyage Distance Calculation

Voyage distance for regulatory and operational purposes is the port-to-port distance sailed. It is taken from logbook, noon reports, AIS/ECDIS tracks, or, for estimation, the planned rhumb-line / great-circle route.

Formula

Distance is the integral of speed over time; for discrete legs it is summed:

D = Integral( V(t) dt ) ~= Sum_legs ( V_leg x t_leg ) Rhumb-line distance d (medium ranges): d = sqrt( (Delta_lon x cos(lat_m))^2 + (Delta_lat)^2 ) x 60

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| V(t) | Speed over ground as a function of time | kn | logged or AIS-derived |
| t_leg | Time on each leg | h | between noon reports or waypoints |
| Delta_lat / Delta_lon | Latitude / longitude difference between waypoints | degrees | decimal degrees |
| lat_m | Mean latitude between waypoints | degrees | for rhumb-line approximation |
| 60 | Conversion 1 degree = 60 nm | nm/deg | on a spherical Earth |

Units

Distance in nautical miles (1 nm = 1.852 km). For regulatory reporting, actual sailed distance is used, not the plan.

Worked Example

Illustrative rhumb-line between two waypoints: lat1 36.0, lon1 -5.0; lat2 45.0, lon2 8.0:

Delta_lat = 9.0 deg ; Delta_lon = 13.0 deg ; lat_m = 40.5 deg

Easting = 13.0 x cos(40.5) = 13.0 x 0.7604 = 9.885 deg

d = sqrt(9.885^2 + 9.0^2) x 60 = sqrt(97.7 + 81) x 60 = sqrt(178.7) x 60

= 13.37 x 60 = 802 nm  (rhumb-line approximation)

Practical Use

Distance is a denominator in CII/EEOI/AER and a driver of fuel and emissions estimates.

- Regulatory reporting (DCS/MRV/CII) — actual sailed distance.

- Voyage planning and ETA estimation.

- Charter-party performance (distance vs CP warranty).

Limitations

- Rhumb-line approximations diverge over long distances — use great-circle for ocean passages.

- Actual sailed distance exceeds the plan due to weather, current and traffic deviation.

- Noon-report distance is human-recorded and subject to rounding.

Official Source

- IMO MEPC.1/Circ.684 — EEOI (distance basis).

- Bowditch — The American Practical Navigator (distance calculation).

- IHO S-52 / ECDIS route-distance standards.

# 13. Great Circle Distance (Haversine / Spherical Law of Cosines)

The shortest distance between two points on the Earth's surface is the great-circle arc. The Haversine formula is numerically stable for small distances; the spherical law of cosines is preferred for well-separated points.

Formula

Haversine formula (Earth as sphere, R = 3440.065 nm):

a = sin^2(Delta_lat/2) + cos(lat1) x cos(lat2) x sin^2(Delta_lon/2) c = 2 x atan2( sqrt(a), sqrt(1-a) ) d = R x c (R = 3440.065 nm = 6371 km)

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| lat1, lat2 | Latitudes of points 1 and 2 (radians) | rad | convert degrees x pi/180 |
| Delta_lat, Delta_lon | Latitude / longitude differences (radians) | rad | in radians |
| R | Mean Earth radius | nm | 3440.065 nm (spherical model) |
| d | Great-circle distance | nm | shortest surface distance |

Units

Result in nautical miles (R=3440.065) or kilometres (R=6371). Angles in radians.

Worked Example

Illustrative — Rotterdam (51.95 N, 4.03 E) to New York (40.67 N, -74.04 W):

lat1 = 0.9075 rad, lat2 = 0.7102 rad

Delta_lat = -0.1973 rad ; Delta_lon = -1.3624 rad

a = sin^2(-0.0987) + cos(0.9075)cos(0.7102)sin^2(-0.6812)

~= 0.00973 + 0.6162 x 0.7583 x 0.3863 ~= 0.00973 + 0.1805 = 0.1902

c = 2 x atan2(sqrt(0.1902), sqrt(0.8098)) = 2 x 0.4469 = 0.8938 rad

d = 3440.065 x 0.8938 = 3,074 nm

Practical Use

Great-circle distance is the planning baseline for ocean passages and for emissions allocation across long legs.

- Voyage planning (shortest route over ocean).

- Long-leg distance input to ETA and consumption estimates.

- Geographic allocation in emissions reporting.

Limitations

- Treats Earth as a sphere; WGS-84 ellipsoidal (Vincenty) differs by up to ~0.5%.

- Does not account for navigational constraints (land, ice, traffic separation).

- Composite sailing is needed where the great-circle exceeds the limiting latitude.

Official Source

- Sinnott — Haversine formula (Sky & Telescope, 1984).

- Vincenty — geodetic inverse formula (WGS-84 ellipsoid).

- Bowditch — The American Practical Navigator.

# 14. ETA Estimation

Estimated time of arrival combines the remaining sailing distance, planned service speed, and an allowance for weather and manoeuvring. It is the operational counterpart to distance and speed calculations.

Formula

Time-to-go and ETA:

TTG = D_remaining / V_planned ETA = Now + TTG + t_allowance t_allowance = t_weather + t_pilot + t_manoevering + t_buffer

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| D_remaining | Remaining distance to destination | nm | great-circle or planned route |
| V_planned | Planned service (order) speed | kn | may be weather-adjusted |
| TTG | Time to go (pure sailing time) | h | D_remaining / V_planned |
| t_allowance | Non-sailing time allowance | h | weather, pilot, manoeuvring, buffer |
| Now | Current date-time of estimate | datetime | UTC consistent |

Units

TTG in hours; ETA in UTC date-time. Allowances are operator judgement and depend on trade and season.

Worked Example

Illustrative — at 06:00 UTC, 1,820 nm remaining, order speed 14.0 kn, allowance 6.0 h (weather + pilot + manoeuvre):

TTG = 1,820 / 14.0 = 130.0 h = 5 days 10 h

ETA = 06:00 + 130.0 h + 6.0 h = 06:00 + 136 h

= day +5 18:00 UTC  (i.e. 5 days, 6h after 06:00 -> 5 days + 12h = day+5 18:00)

Practical Use

Voyage reporting to charterers, just-in-time arrival and port-slot booking.

- Charterer and agent ETA reporting.

- Just-in-time arrival (slow-steaming to slot).

- Schedule recovery / reserve-speed decisions.

Limitations

- Weather and current effects change V_planned en route; ETA drifts.

- Allowances are heuristic and trade-specific.

- CP ETA terms (good-weather, customary despatch) differ from pure arithmetic.

Official Source

- BIMCO standard voyage-reporting practice.

- NYPE 2015 / BIMCO time-charter forms (speed/consumption/ETA warranties).

# 15. Voyage Performance Calculations

Voyage performance synthesises speed, consumption, distance and time into a single review of how the ship performed against plan and against charter-party warranties. It is the basis of claims and off-hire discussions.

Formula

Core voyage-performance ratios:

Average speed (sailed) = D_sailed / t_underway Voyage consumption rate = FC_actual / t_underway Performance ratio = ( D_sailed / V_warranty ) vs FC_allowed FC_allowed = ( t_underway x FC_cp ) + ( t_idle x FC_idle_cp )

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| D_sailed | Actual distance sailed | nm | logbook / AIS |
| t_underway | Underway hours | h | engines on / making way |
| FC_actual | Fuel actually consumed (ME + AE) | t | sounded / MFM |
| FC_cp | Charter-party warranted consumption at the relevant speed | t/day | from CP speed/consumption clause |
| t_idle | Idle / port hours (AE only) | h | port-stay consumption |

Units

Speed in knots, consumption in t/day or t/voyage. Performance ratios are dimensionless or in t (over/under-consumption).

Worked Example

Illustrative — 5,200 nm sailed in 380 h underway; actual FC 1,560 t; CP warranty 14.0 kn on 38 t/day:

Average speed = 5,200 / 380 = 13.68 kn  (vs 14.0 kn -> slightly under)

CP-allowed underway consumption = 380/24 x 38 = 601.7 t  ...

(note: CP consumption is usually expressed per day at sea; compare actual t/day)

Actual rate = 1,560 / (380/24) = 98.5 t/day

Performance review: investigate speed shortfall vs consumption -> hull/propeller or weather.

Practical Use

Post-voyage reconciliation against charter-party warranties and operational plan.

- Charter-party performance / claims evaluation.

- Hull and propeller performance trending.

- Voyage-cost actual-vs-budget review.

Limitations

- CP warranties are 'good weather' — apply good-weather test before drawing conclusions.

- Speed over ground differs from speed through water (current).

- Sounding-based consumption carries measurement uncertainty.

Official Source

- BIMCO / NYPE time-charter forms (speed/consumption warranties).

- English Court arbitration precedent on good-weather and under-performance claims.

# 16. Speed vs Fuel Consumption (Cubic Law & Admiralty)

For displacement ships, propulsive power (and hence main-engine fuel) scales approximately with the cube of speed. This cubic law underlies slow-steaming savings and consumption warranties at different speeds.

Formula

Admiralty / cubic-law relationships:

P ~ V^3 (displacement hull, fixed displacement) FC ~ V^3 (since FC ~ P at near-constant SFC) FC_2 / FC_1 = ( V_2 / V_1 )^3 Admiralty coefficient C = ( Delta^(2/3) x V^3 ) / P

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| P | Propulsive power | kW | main-engine delivered |
| V | Ship speed | kn | through water, calm conditions |
| FC | Main-engine fuel consumption | t/day | approximately ~ P x SFC |
| Delta | Ship displacement | t | fixed for the comparison |
| C | Admiralty coefficient (efficiency indicator) | — | higher = more efficient |

Units

Ratios are dimensionless. The cubic law is an approximation; the real speed-power curve is steeper near the design speed and in heavy weather.

Worked Example

Illustrative — service speed 14.0 kn consumes 38 t/day. Speed reduced to 12.0 kn (same draft):

FC_2 = FC_1 x (V_2/V_1)^3 = 38 x (12.0/14.0)^3

= 38 x (0.857)^3 = 38 x 0.6297 = 23.9 t/day

Daily saving = 38 - 23.9 = 14.1 t/day (approx 37%)

Trade-off: voyage time rises from D/14 to D/12 (longer). Per-voyage fuel = rate x days.

Practical Use

Slow-steaming economics, consumption warranties at varying speeds, and emissions reduction planning.

- Optimum-speed analysis (min cost or min emissions per voyage).

- Charter-party consumption warranties at reduced speed.

- CII/EEXI mitigation via power limitation (EPL/ShaPoLi).

Limitations

- Cubic exponent is approximate; actual exponent varies 2.5-3.5 with hull form and condition.

- Auxiliary consumption does not follow the cubic law (roughly constant with speed).

- At very low speeds the cubic law overstates savings (engine minimum-load floor).

Official Source

- SNAME — Marine engineering practice (speed-power scaling).

- ITTC — Recommended Procedures (performance prediction).

- IMO MEPC.1/Circ.684 — operational efficiency context.

# 17. Weather Correction Concepts

Weather correction removes the effect of wind, waves and current from observed speed/consumption so that ship performance can be compared with the calm-water warranty. ISO 15016 specifies the standard method; the Admiralty and trial-data approaches give simpler approximations.

Formula

Conceptual weather correction (ISO 15016 is the full method):

V_still = V_observed + Delta_V_weather + Delta_V_current Delta_V_weather = f(Beaufort, wave height, heading) Delta_V_current = V_current x cos(angle)

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| V_still | Speed the ship would have made in calm water, no current | kn | comparable with CP warranty |
| V_observed | Speed over ground (or through water) actually sailed | kn | from logbook / AIS |
| Delta_V_weather | Speed lost to wind and waves | kn | per ISO 15016 model or empirical |
| Delta_V_current | Speed gained/lost due to current | kn | current x cos(relative angle) |
| Beaufort, wave height, heading | Sea-state and relative-heading inputs | various | from noon reports |

Units

Speeds in knots. ISO 15016 uses a force-balance (wind, wave-added resistance) approach; simplified Admiralty methods apply percentage allowances (e.g. allowance per Beaufort number).

Worked Example

Illustrative — observed 12.5 kn SOG; following current +0.4 kn; weather penalty -0.3 kn (Beaufort 5 head sea approx):

V_still = 12.5 - 0.4 (subtract following current) + 0.3 (add back weather loss)

= 12.4 kn  (calm-water, still-water equivalent for CP comparison)

Full ISO 15016 applies added-resistance models; this is the conceptual basis only.

Practical Use

Charter-party performance evaluation requires calm-water speed; weather correction produces the comparable figure.

- Good-weather test for speed/consumption claims (BIMCO/NYPE).

- Hull-performance trending (corrected for weather).

- Voyage-estimate accuracy improvement.

Limitations

- Full ISO 15016 is data-intensive (wind, waves, current, heading, trim); simplified methods are approximate.

- CP 'good weather' definitions vary by contract (e.g. Beaufort ceiling, no adverse current).

- Correction quality depends on noon-report sea-state data quality.

Official Source

- ISO 15016 — Guidelines for the assessment of speed and power performance by analysis of trial data.

- ITTC — Recommended Procedures 7.5-04-01-01.1 (speed/power analysis).

- BIMCO / NYPE charter-party good-weather clauses.

# 18. Engine Load Calculations

Engine load is the fraction of maximum continuous rating at which the engine operates. It governs specific fuel consumption, emissions and mechanical wear, and is central to EPL/ShaPoLi and slow-steaming analysis.

Formula

Load factor and effective power:

Load factor (LF) = P_output / MCR P_output = LF x MCR Torque x rpm relation: P = ( T x n ) / 9.549

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| P_output | Instantaneous power output | kW | shaft or electrical |
| MCR | Maximum continuous rating (nameplate or limited) | kW | may be reduced by EPL/ShaPoLi |
| LF | Load factor (fraction of MCR) | — | 0-1 (0-100%) |
| T | Torque | N·m | shaft torque |
| n | Rotational speed | rpm | engine or shaft |

Units

Load factor dimensionless. EEDI/EEXI use 75% MCR as the reference load. Very low load (< ~25-30%) raises SFC and can breach engine operational limits.

Worked Example

Illustrative — engine MCR 12,000 kW, operating at 8,400 kW:

Load factor = 8,400 / 12,000 = 0.70 (70%)

With ShaPoLi limiting MCR to 9,000 kW: LF = 8,400 / 9,000 = 0.933 (93.3%)

Note: EEDI/EEXI reference load is 75% of the (limited) MCR.

Practical Use

Engine load drives SFC, emissions and the basis of EPL/ShaPoLi certification and slow-steaming economics.

- SFC lookup (SFC varies with load).

- EPL/ShaPoLi compliance (effective MCR and override management).

- Maintenance / wear prediction.

Limitations

- SFC is not constant with load; the SFC curve must be applied.

- Auxiliary engines and boilers have separate load profiles.

- Very low load is inefficient and may be operationally constrained.

Official Source

- Engine manufacturer shop-test data (SFC vs load curves).

- IMO MEPC.245(66) / MEPC.333(76) — 75% MCR reference load.

- SNAME marine engineering practice.

# 19. SFOC — Specific Fuel Oil Consumption

SFOC (also written SFC) is the mass of fuel consumed per unit of mechanical or electrical energy delivered. It is the key engine-efficiency parameter and varies with load, ambient conditions and engine condition.

Formula

Definition and test-bench computation:

SFOC = m_fuel / E_delivered = ( m_fuel ) / ( P x t ) [units: g/kWh]

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| m_fuel | Mass of fuel consumed over the test period | g | measured |
| E_delivered | Energy delivered over the period | kWh | = P x t |
| P | Mean brake power output | kW | shaft or electrical |
| t | Test period | h | shop test or in-service period |

Units

SFOC in g/kWh. Typical service values: ~160-175 g/kWh (slow-speed two-stroke on HFO), ~180-210 g/kWh (medium-speed), ~200-230 g/kWh (auxiliary diesels). Always confirm against the engine's specific load curve.

Worked Example

Illustrative — over a 4-hour trial, an engine delivered 9,800 kW average and burned 6,720 kg of fuel:

E_delivered = 9,800 x 4 = 39,200 kWh

m_fuel = 6,720 kg = 6,720,000 g

SFOC = 6,720,000 / 39,200 = 171.4 g/kWh

Practical Use

SFOC is the engine-efficiency input to EEDI/EEXI, fuel-consumption estimates and emissions calculations.

- EEDI/EEXI input (certified SFC from NOx Technical File).

- Pre-voyage fuel-consumption estimation (Topic 11).

- Condition-monitoring of engine performance.

Limitations

- SFOC varies with load — use the value at the relevant load factor.

- Manufacturer values are shop-test; in-service SFOC is typically higher.

- Dual-fuel engines have separate gas/pilot-fuel SFOC curves.

Official Source

- Engine manufacturer NOx Technical File (certified SFC).

- IMO MEPC.245(66) — EEDI guidelines (SFC basis).

- ISO 3046 — Reciprocating internal combustion engines (performance).

# 20. Carbon Intensity Metrics — Consolidated Comparison

Carbon intensity expresses CO2 per unit of transport work or per unit of energy. Multiple variants exist for different purposes (regulatory vs voluntary, design vs operational, tank-to-wake vs well-to-wake). This topic consolidates them.

Formula

Intensity metric definitions:

EEOI = CO2 / ( Cargo x D ) [gCO2/t-nm] AER = CO2 / ( DWT x D_annualised ) [gCO2/dwt-nm] CII = CO2 / ( Capacity x D ) [gCO2/dwt-nm or /GT-nm] EEDI = CO2_design / ( Capacity x V_ref ) [gCO2/t-nm, design] EEXI = same as EEDI, applied to existing ships FuelEU = GHG / Energy [gCO2eq/MJ, well-to-wake]

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| CO2 | Mass of CO2 (sum of fuel x CF) | g | tank-to-wake unless stated |
| Cargo | Cargo carried | t (TEU/pax) | per voyage or period |
| D | Distance | nm | actual sailed |
| D_annualised | V_design x 24 x days | nm | AER basis |
| Energy | Fuel energy content (mass x LCV) | MJ | FuelEU basis |
| GHG | CO2-equivalent mass (CO2 + CH4 + N2O) | gCO2eq | FuelEU; GWP-100 |

Units

Intensity units vary: gCO2/t-nm (EEOI), gCO2/dwt-nm (AER/CII), gCO2/t-nm design (EEDI/EEXI), gCO2eq/MJ (FuelEU). Direct comparison across metrics requires unit reconciliation.

Worked Example

Illustrative alignment of a single voyage across metrics (45,000 t cargo, 50,000 DWT, 4,200 nm, 5,605.2 t-CO2):

EEOI  = 5.6052e9 g / (45,000 x 4,200) = 29.66 gCO2/t-nm

CII-style = 5.6052e9 g / (50,000 x 4,200) = 26.69 gCO2/dwt-nm (per-voyage; CII is annual)

AER would annualise the denominator differently -> not directly comparable to the per-voyage value.

Practical Use

Choosing the correct metric for each audience (regulator, charterer, financier, internal).

- Regulatory: CII (IMO), FuelEU (EU), EEXI/EEDI (IMO design).

- Voluntary: AER (Poseidon Principles, Sea Cargo Charter).

- Operational: EEOI (in-voyage efficiency).

Limitations

- Different denominators (cargo vs capacity vs energy) make cross-metric comparison misleading without reconciliation.

- Tank-to-wake (CII/MRV) vs well-to-wake (FuelEU) scopes differ.

- Annual (CII/AER) vs per-voyage (EEOI) bases differ.

Official Source

- IMO MEPC.1/Circ.684 — EEOI methodology.

- IMO Resolution MEPC.336(76) — CII guidelines.

- Sea Cargo Charter / Poseidon Principles — AER guidance.

- Regulation (EU) 2023/1805 — FuelEU intensity metric.

# 21. Port Stay Calculations

Port-stay calculations determine the time a ship spends in port for loading, discharging and idle periods, and the fuel and emissions associated with auxiliary-engine and boiler consumption during that time. They underpin demurrage and just-in-time arrival.

Formula

Port time and in-port consumption/emissions:

t_port = t_arrival_to_NOR + t_ops + t_completion_to_dep FC_port = ( t_ae x P_ae x SFC_ae ) + ( t_boiler x FC_boiler_rate ) CO2_port = FC_port x CF

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| t_port | Total time in port | h | from arrival to departure |
| NOR | Notice of Readiness tendered | datetime | starts laytime (per CP) |
| t_ops | Cargo operations time | h | loading + discharging |
| P_ae | Auxiliary engine power in port | kW | reefer/crane/hotel load |
| SFC_ae | Auxiliary engine SFC | g/kWh | Topic 19 |
| FC_boiler_rate | Boiler fuel rate | t/h | if applicable |

Units

Time in hours; fuel in tonnes; CO2 in tonnes. Laytime and demurrage follow charter-party terms (e.g. BIMCO Gencon). Shore-power (OPS) displaces in-port fuel (relevant to FuelEU OPS requirements).

Worked Example

Illustrative — 36 h in port, AE 600 kW at SFC 210 g/kWh, boiler 0.3 t/h for 20 h, fuel MGO (CF 3.206):

FC_AE = 36 x 600 x 210 / 10^6 = 4.536 t

FC_boiler = 0.3 x 20 = 6.000 t

FC_port = 4.536 + 6.000 = 10.536 t

CO2_port = 10.536 x 3.206 = 33.78 t-CO2

Practical Use

Port-stay time and consumption feed demurrage/despatch, total-voyage emissions and just-in-time optimisation.

- Laytime / demurrage calculation (BIMCO Gencon).

- In-port emissions reporting (MRV/FuelEU berth term).

- Just-in-time arrival and OPS planning.

Limitations

- Laytime rules (excepted periods, NOR validity) are contract-specific.

- Reefer / cargo-heating loads vary greatly by trade.

- OPS availability changes in-port consumption to zero fuel.

Official Source

- BIMCO Gencon — general voyage charter (laytime & demurrage).

- Regulation (EU) 2023/1805 — FuelEU (berth/OPS requirements).

- IMO MEPC.1/Circ.684 — port-period emissions context.

# 22. Off-Hire Calculations

Off-hire is the period during a time charter when the ship is unavailable due to breakdown, damage or other agreed cause, and hire ceases to be payable. The calculation determines the time and consumption adjustments under the charter-party off-hire clause.

Formula

Standard off-hire time and consumption adjustment:

t_offhire = t_from_event + t_to_recovery + t_underway_loss (per CP) Hire saved = t_offhire x Hire_rate Consumption credit = FC_offhire_event x CP_allocation Net claim = Hire saved +/- consumption / bunker adjustments

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| t_offhire | Off-hire period (per CP clause) | d / h | event to recovery, less excepted periods |
| Hire_rate | Daily time-charter hire rate | USD/d | from CP |
| FC_offhire_event | Fuel consumed during the off-hire event | t | per CP off-hire clause |
| CP_allocation | Consumption/allocation rule in the off-hire clause | — | varies by form (NYPE, Baltime) |

Units

Time in days/hours; hire in currency; consumption in tonnes. Off-hire clauses differ by charter form (NYPE 2015, Baltime, BIMCO Time Charter) — apply the specific wording.

Worked Example

Illustrative — main-engine breakdown 18 h; hire USD 12,000/day; off-hire event fuel 8 t; charterer's bunkers:

t_offhire = 18 h = 0.75 d

Hire saved = 0.75 x 12,000 = USD 9,000

Consumption adjustment per CP (illustrative): owner credits 8 t at the CP value

Net claim = USD 9,000  +  bunker adjustment  (per specific clause wording)

Practical Use

Off-hire calculations settle the hire and bunker adjustments when a ship becomes unavailable under time charter.

- Time-charter hire reconciliation (NYPE, Baltime, BIMCO).

- Insurance / loss-of-hire claims.

- Operational availability (KPI) tracking.

Limitations

- Off-hire wording is contract-specific; 'on/off-hire' and 'net loss' approaches differ.

- Consumption credits depend on whether bunkers are owner's or charterer's.

- Underway time lost during recovery may or may not be included per the clause.

Official Source

- NYPE 2015 — time charter (off-hire clause, lines 17-21).

- BIMCO Time Charter / Baltime — off-hire provisions.

- English Court arbitration precedent on off-hire interpretation.

# 23. Charter Performance Metrics

Charter performance metrics quantify how a chartered ship performed against the contractual speed, consumption and other warranties. They are the basis of under-performance claims and performance bonuses.

Formula

Key charter-performance formulas:

Speed performance = V_actual_good-weather / V_cp_warranty Consumption performance = FC_actual / FC_cp_allowed Time on hire used = t_underway + t_idle (+ off-hire adjustments) Performance claim = ( D / V_cp ) x FC_cp_day - FC_actual [good-weather basis]

Variable Definitions

| Symbol | Variable | Unit | Notes |
| --- | --- | --- | --- |
| V_actual_good-weather | Calm-water speed actually achieved (weather-corrected) | kn | Topic 17 |
| V_cp_warranty | Charter-party warranted speed (good weather) | kn | from CP |
| FC_cp_allowed | CP-allowed consumption for the good-weather underway time | t | warranted rate x days |
| FC_cp_day | Warranted daily consumption | t/day | at the warranted speed |
| D | Good-weather distance sailed | nm | only good-weather legs count |

Units

Speed and consumption ratios dimensionless (or in %). Claim quantities in tonnes of fuel or in hire-day equivalents. All evaluations are on a good-weather basis unless the CP allows alternative wording.

Worked Example

Illustrative — CP warranty 14.0 kn on 38 t/day. Good-weather legs: 3,000 nm in 222 h; actual consumption 360 t:

V_actual_good-weather = 3,000 / 222 = 13.51 kn  (vs 14.0 -> under by 0.49 kn, ~3.5%)

CP-allowed time = 3,000 / 14.0 = 214.3 h = 8.93 d

CP-allowed consumption = 8.93 x 38 = 339.3 t

Over-consumption = 360 - 339.3 = 20.7 t  (approx, before detailed good-weather test)

Performance claim (charterer) ~ 20.7 t, subject to CP terms and arbitration practice.

Practical Use

Charter performance metrics settle under-performance claims and inform charterer fleet-selection decisions.

- Charter-party under/over-performance claims.

- Owner performance-bonus / penalty assessment.

- Charterer fleet-performance benchmarking.

Limitations

- Good-weather definitions and current treatment vary by CP and arbitral precedent.

- Requires reliable weather data and noon reports over sufficient legs.

- Sounding-based consumption introduces measurement uncertainty.

Official Source

- NYPE 2015 / BIMCO Time Charter — speed/consumption warranties.

- English Court arbitration precedent (e.g. The Didymi, The Gas Enterprise).

- BIMCO standard reporting and performance-evaluation practice.