C O M P L I A N C E B R I E F I N G Maritime Regulatory Compliance IMO, EU & SOLAS Frameworks — Requirements, Calculations & Software Automation Prepared for: Poseidon Ledger Scope: IMO DCS · EU MRV · EU ETS · FuelEU · CII · EEXI · ISM · ISPS · MARPOL · SOLAS Reference date: July 2026 Poseidon Ledger Maritime Compliance Series

Executive Summary

International shipping carries roughly 90% of global trade and is regulated through a layered regime of instruments issued by the International Maritime Organization (IMO) and, increasingly, by regional bodies such as the European Union. This report consolidates ten of the most operationally significant instruments — IMO DCS, EU MRV, EU ETS, FuelEU Maritime, CII, EEXI, the ISM Code, the ISPS Code, MARPOL and SOLAS — and, for each, explains its purpose, the documents and calculations it demands, the underlying data, the reporting cadence, the mistakes that most often lead to non-conformity, and the specific ways in which software can automate compliance.

Two themes run through the analysis. First, the regulatory centre of gravity has shifted decisively toward measurable, verifiable environmental performance: where SOLAS, MARPOL, ISM and ISPS established a safety and pollution-prevention floor, CII, EEXI, EU ETS and FuelEU Maritime now impose a data-intensive ceiling on emissions and energy intensity. Second, the volume, granularity and cross-referencing of mandatory data has grown beyond what manual spreadsheet workflows can reliably sustain, making digital data pipelines — not additional headcount — the decisive compliance asset.

The report concludes that a single, integrated data backbone feeding every instrument from one verified source reduces both cost and risk: emissions data captured once for EU MRV can serve IMO DCS, EU ETS surrender, CII rating and FuelEU intensity calculations, while the same document-management discipline that underpins ISM and ISPS provides the evidentiary chain that auditors and verifiers expect. The chapter on cross-regulation analysis and software automation maps these overlaps explicitly.

Table of Contents

1. Regulatory Landscape and How the Instruments Interact	2

2. IMO Data Collection System (DCS)	2

Purpose	3

Required Documents	3

Required Calculations	4

Required Data	4

Reporting Frequency	5

Common Compliance Mistakes	5

How Software Can Automate Compliance	6

3. EU MRV (Monitoring, Reporting, Verification of CO2)	7

Purpose	7

Required Documents	7

Required Calculations	8

Required Data	9

Reporting Frequency	9

Common Compliance Mistakes	9

How Software Can Automate Compliance	10

4. EU Emissions Trading System (ETS) — Maritime	11

Purpose	11

Required Documents	11

Required Calculations	12

Required Data	13

Reporting Frequency	13

Common Compliance Mistakes	14

How Software Can Automate Compliance	14

5. FuelEU Maritime	15

Purpose	15

Required Documents	16

Required Calculations	16

Required Data	17

Reporting Frequency	18

Common Compliance Mistakes	18

How Software Can Automate Compliance	19

6. Carbon Intensity Indicator (CII)	20

Purpose	20

Required Documents	20

Required Calculations	21

Required Data	22

Reporting Frequency	22

Common Compliance Mistakes	22

How Software Can Automate Compliance	23

7. Energy Efficiency Existing Ship Index (EEXI)	24

Purpose	24

Required Documents	24

Required Calculations	25

Required Data	26

Reporting Frequency	26

Common Compliance Mistakes	26

How Software Can Automate Compliance	27

8. ISM Code (International Safety Management)	28

Purpose	28

Required Documents	29

Required Calculations	29

Required Data	30

Reporting Frequency	30

Common Compliance Mistakes	31

How Software Can Automate Compliance	31

9. ISPS Code (International Ship and Port Facility Security)	32

Purpose	32

Required Documents	33

Required Calculations	33

Required Data	34

Reporting Frequency	34

Common Compliance Mistakes	35

How Software Can Automate Compliance	35

10. MARPOL (International Convention for the Prevention of Pollution from Ships)	36

Purpose	37

Required Documents	37

Required Calculations	38

Required Data	38

Reporting Frequency	39

Common Compliance Mistakes	39

How Software Can Automate Compliance	40

11. SOLAS (International Convention for the Safety of Life at Sea)	41

Purpose	41

Required Documents	41

Required Calculations	42

Required Data	43

Reporting Frequency	43

Common Compliance Mistakes	43

How Software Can Automate Compliance	44

12. Cross-Regulation Analysis and Software Automation	45

12.1 The Shared Data Layer	45

12.2 Overlapping Obligations to Reconcile	46

12.3 What a Compliance Platform Should Do	46

13. Conclusions and Recommendations	47

Note: This Table of Contents is generated via field codes. To ensure page-number accuracy after editing, please right-click the TOC and select "Update Field."

# 1. Regulatory Landscape and How the Instruments Interact

Maritime regulation is best understood as two concentric layers. The inner layer — SOLAS, MARPOL, the ISM Code and the ISPS Code — is the mature, global safety-and-environment baseline enforced through flag States, port States and Recognized Organizations. The outer layer, which has expanded rapidly since 2018, is the climate and energy regime: IMO DCS, CII and EEXI at the global level, and EU MRV, EU ETS and FuelEU Maritime as the European region's market-based and technology-forcing instruments.

Although each instrument is legally distinct, they share most of the same raw data. Fuel consumption, distance travelled, hours underway, cargo carried and emission-factor values feed IMO DCS, EU MRV, CII, EU ETS and FuelEU Maritime simultaneously. The conventions differ in scope (global versus EU), unit (well-to-wake versus tank-to-wake), geography (100% of EU voyages versus 50% of inbound/outbound voyages), and mechanism (reporting, index, rating, allowance surrender or penalty). Understanding these overlaps is the prerequisite for building a compliance system that captures data once and reports it many ways.

Table 1: Quick-reference comparison of the ten instruments covered in this report

| Instrument | Source | Primary mechanism | Core metric |
| --- | --- | --- | --- |
| IMO DCS | IMO (MARPOL VI) | Annual fuel-data reporting | tCO2 per voyage/year |
| EU MRV | EU (Reg. 2015/757) | Verified annual emissions report | tCO2, transport work |
| EU ETS | EU (Dir. 2003/87) | Allowance surrender (market-based) | tCO2 / EUA price |
| FuelEU Maritime | EU (Reg. 2023/1805) | GHG-intensity limit + penalty | gCO2eq/MJ (WtW) |
| CII | IMO (MARPOL VI) | Annual rating A–E | gCO2/dwt·nm |
| EEXI | IMO (MARPOL VI) | One-off technical index | EEXI vs required |
| ISM Code | IMO (Res. A.741) | Safety-management system | SMS conformity |
| ISPS Code | IMO (Res. A.974) | Ship & port security plans | Security level readiness |
| MARPOL | IMO (6 Annexes) | Operational discharge limits | Annex-specific |
| SOLAS | IMO (Chapters) | Construction & equipment | Equipment/structural |

Each subsequent chapter follows an identical structure — purpose, documents, calculations, data, frequency, common mistakes and software automation — so that the instruments can be compared on a like-for-like basis. Where a value, threshold or date could change, it is marked as indicative and the reader is directed to the issuing body for the current text.

# 2. IMO Data Collection System (DCS)

The IMO Data Collection System, in force since 1 March 2018 under MARPOL Annex VI, requires ships of 5,000 GT and above to record and report their annual fuel-oil consumption to their Administration. It was the first global, mandatory mechanism for collecting ships' fuel-use data and underpins the IMO's subsequent short-term greenhouse-gas measures, including CII.

## Purpose

DCS exists to give the IMO a verified, ship-level evidence base of aggregate fuel consumption and CO2 emissions. That base is used to track progress against the IMO Initial and Revised GHG Strategies (notably the 2030 and 2040 indicative checkpoints and the 2050 net-zero goal), to calibrate future measures, and to ensure each ship is accountable for the data it produces.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Record of fuel consumption: the underlying shipboard log of fuel used per voyage and per year, by bunker type.

- DCS dataset (IMO format): the structured data record submitted to the Administration, covering consumption, distance, hours underway and method.

- Statement of Compliance (SoC): issued by the Administration or its Recognized Organization after verification, valid for the calendar year reported.

- SEEMP (Part I): the Ship Energy Efficiency Management Plan must be on board and referenced by the DCS process.

## Required Calculations

DCS itself is primarily a data-collection mechanism, but a small number of determinations must be made for the record to be valid:

- Annual fuel consumption by type: aggregation of HFO/VLSFO/MGO/LNG and other fuels, in metric tonnes, per the bunker delivery notes and tank soundings.

- Distance travelled: nautical miles from departure to arrival, taken from the logbook or an automatic recording device.

- Hours underway: engine-running or underway time used to derive average speed and to cross-check consumption.

- CO2 estimate (supporting): although DCS reports fuel mass, CO2 is commonly derived using IMO emission factors to align with downstream uses (CII, EU MRV).

## Required Data

The following data elements must be captured to support reporting and verification:

- Bunker delivery notes (BDN)   •   Tank-sounding / mass flow meter (MFM) readings   •   Voyage legs (port-to-port)   •   Distance and hours underway   •   Fuel type / sulphur content   •   Ship particulars (dwt, GT, IMO number)

## Reporting Frequency

Reporting is annual. Each calendar year's data is consolidated, verified by the Administration or its RO, and a Statement of Compliance is issued for the following year. The first full reporting year was 2019.

## Common Compliance Mistakes

- Reconciling MFM totals against BDNs only at year-end, when discrepancies are already embedded in the record.

- Excluding voyages that start or end outside the reporting calendar, creating gaps that verifiers flag.

- Using inconsistent fuel categorisation between the engine-room log and the DCS dataset.

- Leaving the SEEMP Part I out of date, so the data-collection methodology on file does not match what the crew actually does.

- Treating DCS as a pure reporting exercise and missing that its data drives the ship's CII rating.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- Automated capture of MFM and sounding data into a single consumption ledger, reconciled against each BDN on delivery.

- Voyage detection from AIS/ECDIS that auto-creates port-to-port legs with distance and underway hours.

- A pre-configured IMO DCS export that maps the verified ledger to the required dataset fields.

- Continuous CII projection from the same data, so the crew sees the rating impact of consumption in real time.

- Document vaulting of the SoC and SEEMP with expiry alerts ahead of the annual cycle.

# 3. EU MRV (Monitoring, Reporting, Verification of CO2)

EU MRV — Regulation (EU) 2015/757 — was the European Union's first mandatory shipping-emissions reporting regime. Since 1 January 2018 it has required ships above 5,000 GT calling at EU/EEA ports to monitor, report and have independently verified their CO2 emissions, fuel consumption and transport work. It was substantially amended in 2023 to expand its scope (adding CH4 and N2O, and smaller and offshore ships in stages) and to align it with the EU ETS and FuelEU Maritime.

## Purpose

EU MRV creates a transparent, verified emissions record for shipping activity connected to the EU, providing the factual input for EU ETS allowance surrender and for FuelEU Maritime compliance. It also gives the European Commission and the public a reliable picture of the sector's carbon footprint in European trade.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Monitoring Plan: a ship-specific document describing the monitoring methods, emission sources, fuels and procedures; it must be assessed by an accredited verifier before the reporting period.

- Annual Emissions Report (AER): the verified report submitted to the Commission/THETIS-MRV by 30 April, covering emissions, fuel consumption, distance, time at sea, cargo carried and transport work.

- Document of Compliance (DoC): issued by the verifier for each verified report; must be carried on board.

- Verifier's assessment records: the verifier's risk analysis, data-sampling and evidence files supporting the verification opinion.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Emissions per voyage: fuel consumed × emission factor, split into CO2 and (from the amended scope) CH4 and N2O, with voyage-level allocation.

- Geographic allocation: 100% of emissions from voyages and intra-EU/EEA port calls, plus 50% of emissions from voyages into or out of EU/EEA ports.

- Transport work: cargo carried (tonnes or passenger/guest-equivalent) × distance, used to derive emissions intensity (gCO2/t·nm).

- Emission factors: applied per fuel type and, for the expanded scope, per greenhouse gas, on a tank-to-wake basis.

## Required Data

The following data elements must be captured to support reporting and verification:

- Voyage records (port, date, berth time)   •   Fuel consumption per fuel type per voyage   •   Distance sailed and time at sea   •   Cargo carried per voyage   •   Monitoring-method evidence (BDN, MFM, bunker tank)   •   Ship particulars and monitoring-plan reference

## Reporting Frequency

Annual cycle: monitoring throughout the calendar year, the Annual Emissions Report submitted to THETIS-MRV by 30 April of the following year, verified before submission, and the Document of Compliance issued afterwards.

## Common Compliance Mistakes

- Failing to keep the Monitoring Plan in step with on-board reality, so the verifier finds a method mismatch.

- Mis-classifying voyages as intra-EU versus extra-EU, which corrupts the 100%/50% allocation.

- Recording cargo at the wrong point (loading versus sailing) and distorting transport work.

- Leaving CH4/N2O and the smaller-vessel scope changes (post-2023 amendment) out of the Monitoring Plan.

- Submitting the AER before verification, or missing the 30 April deadline and exposing the company to penalties.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- A configurable Monitoring Plan template that is versioned alongside the live data pipeline, evidencing method consistency.

- Automatic voyage classification (intra/extra-EU, at berth, manoeuvring) from AIS and port-call data.

- Per-voyage emission calculation with regulatory emission-factor tables maintained centrally.

- One-click generation of the AER in the THETIS-MRV format, and archiving of the verifier's evidence pack.

- Shared ledger with EU ETS and FuelEU so a single verified dataset feeds all three instruments.

# 4. EU Emissions Trading System (ETS) — Maritime

Shipping was brought into the EU ETS by Directive (EU) 2023/959, amending the ETS Directive (2003/87/EC). From 1 January 2024, large ships (≥ 5,000 GT) trading to, from and between EU/EEA ports must surrender EU Allowances (EUAs) for a share of their reported CO2 emissions. The system turns the emissions already measured under EU MRV into a financial obligation and is the EU's principal market-based measure for the sector.

## Purpose

By putting a price on CO2 emitted on EU-related voyages, EU ETS creates a direct economic incentive to reduce emissions — through efficiency, slow-steaming, alternative fuels or operational change — and integrates shipping into the broader EU carbon market alongside aviation, power and industry.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- EU MRV Annual Emissions Report: the verified AER is the factual basis for the volume of allowances to be surrendered.

- Company account in the Union Registry: each shipping company must open and operate an account to hold, transfer and surrender EUAs; from 2025 this sits under the Maritime Operator Holding Account framework administered through the responsible Member State (or, for non-EU companies, the implementing-administration).

- Surrender records: evidence of EUAs surrendered against the verified obligation each year.

- Verifier's DoC and compliance records: the chain linking verified emissions to the surrendered volume.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Surrender obligation: verified CO2 emissions (from EU MRV), allocated 100% for intra-EU/EEA voyages and at berth, 50% for extra-EU/EEA inbound/outbound voyages.

- Phased coverage: 40% of 2024 emissions surrendered in 2025, 70% of 2025 emissions surrendered in 2026, and 100% of 2026 emissions surrendered in 2027 and thereafter. (Methane and nitrous oxide are brought into scope from 2026 reporting.)

- Allowance cost: surrendered volume × EUA market price; companies must budget for procurement, timing and hedging.

- Ice-class and other adjustments: specific reductions apply for certain ice-class vessels and voyages to outermost regions, where applicable.

## Required Data

The following data elements must be captured to support reporting and verification:

- Verified EU MRV emissions data   •   EU/EEA port-call sequence (geographic allocation)   •   Company and ship identification in the Union Registry   •   EUA holdings and transactions   •   EUA market prices and procurement plan   •   Verifier evidence chain

## Reporting Frequency

Annual surrender: the obligation for a given reporting year is settled by surrendering allowances in the following September (e.g. 2024 emissions are surrendered by September 2025). MRV reporting remains annual, by 30 April. Companies also face quarterly monitoring of their position against expected surrender.

## Common Compliance Mistakes

- Underestimating the 40/70/100% phase-in and being caught short on allowance procurement.

- Confusing the reporting entity (the 'shipping company' as defined) with the registered shipowner or the technical manager.

- Not opening or funding the Union Registry account in time to allow orderly EUA purchases.

- Failing to agree commercial allocation of ETS cost between owner, charterer and sub-charterer in the contract.

- Overlooking the shift to 100% coverage and the addition of CH4/N2O from the 2026 reporting year.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- Live, voyage-level accumulation of the EU ETS surrender exposure straight from the EU MRV ledger.

- Phase-in-aware projection that re-baselines the obligation as the percentage step changes each year.

- Registry integration that reconciles holdings, surrenders and the verified obligation in one view.

- EUA price scenarios and a procurement calendar with alerts ahead of the September deadline.

- Charter-party tagging that allocates ETS cost to the responsible commercial party per voyage line.

# 5. FuelEU Maritime

FuelEU Maritime (Regulation (EU) 2023/1805) entered into force on 1 January 2025. It is a technology-forcing instrument: rather than pricing carbon, it sets a limit on the greenhouse-gas intensity of the energy used on board ships calling at EU/EEA ports, measured on a well-to-wake basis, and it rewards the uptake of sustainable renewable fuels. It applies to the same ≥ 5,000 GT population as EU MRV/ETS (with passenger and offshore expansions on a staged basis).

## Purpose

FuelEU Maritime aims to pull the maritime fuel mix toward low- and zero-carbon alternatives by guaranteeing demand for them. Where EU ETS raises the cost of emitting, FuelEU limits the carbon intensity of energy itself, and imposes a financial penalty on energy used above the limit.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Monitoring Plan (FuelEU): a ship-specific plan defining the energy sources, emission factors (well-to-tank and tank-to-wake) and monitoring methodology.

- FuelEU emissions report: the annual report of energy used and resulting WtW GHG intensity, submitted and verified alongside the EU MRV cycle.

- Document of Compliance (FuelEU): issued by the verifier confirming the limit was met or, where not, that the penalty was paid.

- Proof of sustainable-fuel origin: for biofuels, RFNBOs and other certified pathways, evidence (e.g. sustainability certificates) supporting the emission factors claimed.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Well-to-wake GHG intensity: for each energy carrier, (well-to-tank + tank-to-wake) emission factor, expressed in gCO2eq/MJ, weighted across all energy used.

- Compliance balance: the difference between the ship's achieved intensity and the regulatory limit, multiplied by the energy used; a negative balance indicates non-compliance.

- Limit trajectory: a 2% reduction from the 2020 reference (91.16 gCO2eq/MJ) in 2025, rising to 6% by 2030 and stepping up to 80% by 2050 (values indicative — verify against current text).

- Penalty: applied to the excess energy at roughly €2,400 per tonne of VLSFO-equivalent, escalating for repeat non-compliance through the banking/borrowing mechanism.

- Rewards and incentives: the regulation includes a reward multiplier for RFNBOs (in the early years) and a container on-board connection requirement for shore power where OPS is available.

## Required Data

The following data elements must be captured to support reporting and verification:

- Energy used per fuel type (in MJ)   •   Well-to-tank and tank-to-wake emission factors per fuel   •   Voyage geography (100% EU / 50% extra-EU)   •   On-shore power supply (OPS) use at berth   •   Sustainable-fuel certificates   •   Ship and engine configuration

## Reporting Frequency

Annual, aligned with EU MRV: monitoring throughout the year, the verified FuelEU report and compliance balance by 30 June of the following year, and any penalty settled on the notified schedule. The limit tightens in five-year steps.

## Common Compliance Mistakes

- Confusing tank-to-wake (EU MRV/ETS) with well-to-wake (FuelEU) intensity and double-counting or omitting the upstream component.

- Claiming RFNBO or biofuel emission factors without retaining the supporting sustainability certificates.

- Overlooking the shore-power (OPS) obligation at covered EU ports for applicable container and passenger ships.

- Borrowing from a future compliance period without planning the repayment, leaving the next year non-compliant.

- Treating FuelEU and EU ETS as substitutes rather than two layered obligations on the same activity.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- A fuel-and-energy master ledger with both TtW and WtW factors maintained centrally so a single fuel entry feeds both EU ETS/MRV and FuelEU.

- Automatic computation of the compliance balance and a running projection against the five-year limit trajectory.

- Reward and penalty modelling for fuel-switching scenarios (e.g. LNG, biofuel blends, e-fuels) before procurement.

- Certificate vaulting that links each batch of sustainable fuel to its sustainability evidence and emission factor.

- OPS-use capture and validation against the port infrastructure list to evidence the shore-power obligation.

# 6. Carbon Intensity Indicator (CII)

The CII is the operational-efficiency element of the IMO's short-term GHG measure, introduced by MARPOL Annex VI amendments that took effect on 1 November 2022. For ships of 5,000 GT and above, it expresses each year's carbon intensity as an operational rating and requires a corrective plan when the rating falls below acceptable.

## Purpose

CII translates each ship's annual operation into a single, comparable efficiency grade (A to E), creating an ongoing incentive to reduce emissions per unit of transport work through speed, routing, hull condition, energy -saving devices and fuel choice.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- SEEMP Part III — CII Implementation Plan: for ships rated D or E (or the equivalent for three consecutive years), a documented plan of corrective measures, verified by the RO.

- Verified annual CII record: the computed Attained CII and assigned rating, recorded with the DCS data set.

- Statement of Compliance: reflecting the CII rating for the reporting year.

- Supporting DCS data: CII is derived from the IMO DCS dataset, so the underlying consumption, distance and capacity data must be intact.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Attained CII: annual emissions (derived from fuel × CO2 factor) divided by (capacity × distance); for most ship types, capacity is deadweight, expressed in gCO2/dwt·nm.

- Required CII: the year-specific limit from the ship's reference line and the reduction factor for that year (the limits tighten annually through 2026).

- Rating: A, B, C, D or E, determined by how far the Attained CII deviates from the Required CII; the band edges widen each year, making the same performance harder to maintain.

- Rating-year adjustment: correction factors and voyage exclusions (e.g. for specific ice navigation, safety, or shuttle-tank operations) are applied per the latest guidelines before rating.

## Required Data

The following data elements must be captured to support reporting and verification:

- Annual fuel consumption by type   •   Distance travelled (nm)   •   Capacity (dwt or GT, by ship type)   •   Hours underway   •   Voyage corrections and exclusions   •   Ship type and particulars

## Reporting Frequency

Annual. The Attained CII is computed from the verified DCS data each calendar year, a rating is assigned, and any required SEEMP Part III corrective plan is submitted for the following cycle.

## Common Compliance Mistakes

- Reading CII only at year-end, when there is no time left to correct a slipping rating.

- Mis-applying correction factors or claiming exclusions that the verifier cannot evidence.

- Letting the SEEMP Part III become generic rather than a genuine, ship-specific corrective plan.

- Ignoring the progressive tightening of the Required CII, so a 'C' one year becomes a 'D' the next at unchanged performance.

- Disconnecting CII from commercial decisions (charter speed, laycan) so the rating degrades unnoticed.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- Continuous, in-year CII projection from the live consumption and AIS-derived distance, showing the projected rating any day of the year.

- What-if modelling for speed reduction, weather routing and fuel switches, with their rating impact before commitment.

- Automatic flagging when the projected rating crosses into D or E, triggering SEEMP Part III preparation early.

- Correction-factor and exclusion checking built into the calculation engine, with evidence captured automatically.

- Charterer-facing dashboards so commercial decisions reflect their CII consequence.

# 7. Energy Efficiency Existing Ship Index (EEXI)

The EEXI is the technical (design) counterpart to CII, introduced by the same 2021 MARPOL Annex VI amendments and effective from 1 November 2022. It is a one-off certification that an existing ship's installed power and efficiency attain a required level, broadly analogous to the EEDI applied to new ships.

## Purpose

EEXI ensures that the existing fleet's technical design meets a minimum energy-efficiency threshold, forcing owners of inefficient ships to apply measures — typically engine power limitation (EPL or ShaPoLi) — so that the technical baseline of the whole fleet is raised toward the new-build standard.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Technical File / EEXI File: ship-specific document showing the attained EEXI calculation, the required EEXI and the basis of the calculation, approved by the RO.

- Engine Power Limitation (EPL/ShaPoLi) documentation: where a limiter is needed to comply, the certified limit, its sealing/override arrangements and the onboard management procedure.

- International Energy Efficiency (IEE) Certificate: issued on the basis of the approved EEXI file, evidencing compliance.

- Approved supporting calculations: speed/power trial results, reference speed data and any wind/solar auxiliary-power credit evidence.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Attained EEXI: (installed power × CO2 factor × correction) divided by (capacity × reference speed), aggregated across main and auxiliary engines, with optional innovative-efficiency credits.

- Required EEXI: derived from the applicable EEDI reference line for the ship type and size, multiplied by a reduction factor.

- Power limitation: where Attained EEXI exceeds Required, the level of EPL/ShaPoLi needed to bring it within the limit, with the resulting restricted power recorded.

- Overridable-limit management: the calculation of when override is permitted (emergencies) and the logging that evidences correct use.

## Required Data

The following data elements must be captured to support reporting and verification:

- Main and auxiliary engine power and specific fuel consumption   •   Installed power / MCR   •   Capacity (dwt/GT)   •   Reference speed from speed-power trials   •   Fuel types in use   •   ShaPoLi/EPL setting and override logs

## Reporting Frequency

EEXI is a one-off certification at first survey after the 2022 entry into force, but it must be re-validated whenever a relevant change occurs (e.g. engine change, modification to power limitation, or conversion that affects the index).

## Common Compliance Mistakes

- Letting an overridable limiter be overridden without the mandatory log entry, breaching the certified condition.

- Not re-validating the EEXI after an engine, propeller or hull change that affects the calculation.

- Relying on outdated speed-trial data, so the reference speed no longer reflects the ship's condition.

- Treating EEXI as 'done' because the certificate is on board, while ignoring its interaction with CII in operation.

- Miscalculating auxiliary-power and innovative-technology credits, which verifiers then reject.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- A central asset register of each ship's EEXI file, IEE Certificate and ShaPoLi/EPL settings, with change-triggers for re-validation.

- Override-log monitoring that flags undocumented or excessive use of power-limit override.

- Linkage between the certified EPL/ShaPoLi and operational data, so any breach is detected in near-real time.

- Document workflows for technical-file approval and renewal, with expiry and amendment alerts.

- Scenario modelling for retrofits (hull coating, propeller, energy-saving device) showing the EEXI and CII effect together.

# 8. ISM Code (International Safety Management)

The ISM Code (IMO Resolution A.741, mandatory since 1998) requires every shipping company to develop, implement and maintain a Safety Management System covering ship operations and environmental protection. It is the umbrella management standard that gives every other instrument its operational teeth.

## Purpose

ISM exists to ensure that ships are operated safely, that the environment is protected, and that safety and pollution-prevention are managed through a documented, auditable system with clear responsibility, rather than left to individual practice.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Safety Management System (SMS) manual: the company's documented system, including procedures, responsibilities and emergency preparedness.

- Document of Compliance (Doc): issued to the company by the Administration/RO, confirming the SMS meets the Code; subject to annual verification.

- Safety Management Certificate (SMC): issued to each ship, confirming it is operated in line with the company's approved SMS.

- Records of non-conformities, accidents and drills: objective evidence that the system is being used and improved.

## Required Calculations

Compliance depends on the following quantitative determinations:

- No numeric index: ISM is a management standard, not a calculation; 'compliance math' consists of audit findings, corrective-action closure rates and lagging safety indicators (LTI, near-miss rates).

- Risk assessment: qualitative or semi-quantitative risk ranking used to prioritise tasks and controls within the SMS.

- Corrective-action metrics: tracking of open non-conformities, time-to-close and recurrence, as the management measure of system health.

## Required Data

The following data elements must be captured to support reporting and verification:

- Audit schedules and reports   •   Non-conformity and corrective-action records   •   Accident / incident / near-miss reports   •   Drill and training records   •   Maintenance records linked to safety equipment   •   Master's standing orders and review reports

## Reporting Frequency

The company Document of Compliance is verified annually and re-issued every five years; the ship's SMC is subject to an intermediate and a renewal verification; internal audits and management reviews occur at least annually, and drills on a recurring schedule.

## Common Compliance Mistakes

- Letting the SMS become a paper system disconnected from how the ship is actually run.

- Closing non-conformities without genuine root-cause analysis, so the same finding recurs at the next audit.

- Under-reporting near-misses, losing the early-warning data that the Code is designed to capture.

- Failing to cascade changes in other regulations (CII, FuelEU, cyber) into SMS procedures.

- Letting the Designated Person Ashore (DPA) role become nominal rather than an active assurance function.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- A digital SMS with controlled, version-tracked procedures and acknowledgements by role and vessel.

- An integrated non-conformity and corrective-action workflow with automated root-cause templates and due-date alerts.

- Near-miss and incident capture from mobile devices at sea, feeding dashboards for the DPA and management review.

- Automated audit and drill scheduling that aligns with Doc/SMC cycles and flags overdue items.

- Cross-linking so that findings from environmental regulations (MRV, FuelEU) surface in the SMS review.

# 9. ISPS Code (International Ship and Port Facility Security)

The ISPS Code (IMO Resolution A.974, mandatory since 2004) is the maritime security counterpart to ISM. It requires ships on international voyages and the port facilities serving them to assess security risks and operate under approved security plans at three security levels.

## Purpose

ISPS exists to detect and deter security threats to ships and port facilities — including piracy, terrorism, stowaways and unlawful interference — through a structured, risk-based regime that links ship, company and port facility under a common framework.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Ship Security Plan (SSP): the approved plan detailing measures at each of the three security levels; carried on board and subject to periodic verification.

- International Ship Security Certificate (ISSC): issued after verification that the SSP meets the Code; subject to intermediate and renewal verification.

- Ship Security Assessment (SSA): the underlying risk assessment supporting the SSP, including threat and vulnerability analysis.

- Continuous Synopsis Record (CSR): the onboard record of the ship's history of ownership, flag, company and class, kept up to date.

## Required Calculations

Compliance depends on the following quantitative determinations:

- No numeric index: like ISM, ISPS is a management regime; compliance is evidenced through drill records, security-incident reports and verification findings rather than a calculated metric.

- Security-level readiness: the ship's ability to escalate from Security Level 1 (normal) to Level 2 (heightened) and Level 3 (exceptional) per the SSP, measured by drill performance.

- Access-control and inventory counts: operational counts (persons on board, restricted-area access logs) used as evidence the plan is being applied.

## Required Data

The following data elements must be captured to support reporting and verification:

- Security incident and breach records   •   Drill and exercise records   •   Access-control and visitor logs   •   Threat intelligence / port security advisories   •   Ship Security Officer (SSO) records   •   CSR and flag/company/class history

## Reporting Frequency

The ISSC is verified at intermediate and renewal cycles; security drills must be conducted at least every three months, plus an annual full exercise; the SSP is reviewed and re-approved at least every five years and whenever the SSA changes.

## Common Compliance Mistakes

- Operating permanently at 'Level 1' on paper while not actually applying its access-control measures.

- Treating drills as a tick-box, so escalation to Level 2/3 is untested when genuinely needed.

- Letting the CSR fall out of date after changes of owner, company, flag or class.

- Disconnecting the SSP from cyber-security risks now expected to be covered within the security regime.

- Weak coordination between ship security officers and port facility security officers (PFSOs).

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- Digital SSP with controlled distribution, level-escalation checklists and acknowledgement tracking.

- Electronic access control and visitor logging that produces the evidence verifiers expect without manual collation.

- Drill scheduling and after-action tracking tied to the ISSC verification cycle.

- CSR maintenance workflow that prompts updates on every ownership, company, flag or class change.

- Integration of cyber-risk registers into the SSA so the security plan reflects current threats.

# 10. MARPOL (International Convention for the Prevention of Pollution from Ships)

MARPOL is the foundational environmental convention of the IMO, in force since 1983 and structured in six Annexes covering oil (I), noxious liquid substances in bulk (II), harmful packaged substances (III), sewage (IV), garbage (V) and air pollution (VI). Annex VI is the home of the sulphur cap, the NOx/SOx limits, DCS, CII and EEXI.

## Purpose

MARPOL exists to eliminate or minimise pollution of the marine environment from ship operations — whether by accidental discharge, routine operational discharge, or emissions to air — through uniform international standards enforced by flag and port States.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Oil Record Book (Parts I & II): mandatory log of bunkering, oil transfer and disposal operations under Annex I.

- Cargo Record Book / Procedures and Arrangements Manual: for chemical tankers and ships carrying noxious liquid substances under Annex II.

- Garbage Record Book (Parts I & II) and Garbage Management Plan: required under Annex V for ships of 400 GT and above / ≥ 12 persons.

- International Air Pollution Prevention (IAPP) Certificate and Supplement: evidencing Annex VI compliance, including the SOx/NOx limits and the 0.50% global sulphur cap.

- Engine technical files and bunker delivery notes: supporting Annex VI air-pollution compliance, retained for three years.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Fuel sulphur content: must not exceed 0.50% globally (0.10% in Emission Control Areas); verified from BDNs and spot sampling.

- NOx emission limits: engine-specific limits depending on rated speed and the applicable Tier, per the engine's Technical File.

- Oily-water discharge thresholds: machinery-space bilge water discharge criteria (e.g. 15 ppm oil content with equipment in operation) under Annex I.

- Garbage discharge categories: what may and may not be discharged by distance-from-nearest-land and substance category, under Annex V.

## Required Data

The following data elements must be captured to support reporting and verification:

- Bunker delivery notes (sulphur)   •   Oil Record Book entries   •   Garbage Record Book and receipts   •   Oily-water separator / 15-ppm monitor records   •   Engine NOx Technical File   •   IAPP / IOPP certificates and supplements

## Reporting Frequency

Continuous operational logging (record books maintained at each operation); certificate cycles vary by Annex (annual/intermediate/renewal surveys for IOPP, IAPP and others); BDNs retained for three years.

## Common Compliance Mistakes

- Incorrect entries (or missing entries) in the Oil Record Book, historically the most common port-State detention cause.

- Failing to reconcile BDN sulphur values with actual fuel, especially after bunker switchovers.

- Mis-applying Annex V garbage categories and discharging prohibited waste under the wrong distance rule.

- Letting the 15-ppm monitor be bypassed or inoperable, undoing the equipment-based discharge defence.

- Not updating IAPP/Annex VI documentation after changes that affect EEXI/CII or engine certification.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- Electronic record books for oil, cargo and garbage that enforce mandatory fields and prevent the classic 'missing entry' finding.

- BDN ingestion with automatic sulphur-content validation against the applicable cap (global and ECA), flagging non-compliant fuel.

- Oily-water separator run logging integrated with the 15-ppm monitor so bypasses are detected automatically.

- Survey and certificate calendar across all MARPOL Annexes with expiry and survey-due alerts.

- Central storage of Technical Files, IAPP/IOPP supplements and bunker records ready for port-State inspection.

# 11. SOLAS (International Convention for the Safety of Life at Sea)

SOLAS is the most important of the international treaties concerning ship safety. First adopted in 1914 in the wake of the Titanic disaster, the version in force today is the 1974 Convention (as amended), covering construction, subdivision and stability, machinery, fire protection, life-saving appliances, radiocommunications, navigation, carriage of cargoes and dangerous goods, nuclear ships, and management/operations.

## Purpose

SOLAS exists to specify minimum standards for the construction, equipment and operation of ships, so that human life at sea is protected. Its requirements are enforced by flag States (certification) and port States (inspection) and are continuously updated to address new risks.

## Required Documents

The following documents form the core compliance evidence set for this regulation:

- Class and statutory certificates: Cargo Ship Safety Construction, Equipment and Radio Certificates (with Record of Equipment), Passenger Ship Safety Certificate, and others as applicable.

- Stability and loading information: approved stability booklet, loading instruments and damage-survival information.

- Lifesaving and fire-control plans: carried on board and kept current with the ship's arrangement.

- Navigational and GMDSS records: logbook entries, voyage data recorder (VDR), AIS, and communications records.

- Safety Management Certificate (link to ISM): the operational safety overlay on SOLAS's technical requirements.

## Required Calculations

Compliance depends on the following quantitative determinations:

- Stability and load-line margins: intact and damage-stability criteria, freeboard, and the effect of loading on each.

- Equipment and survival capability: lifeboat/rescue-boat capacity, davit loads, and fire-main/extinguishing sizing against SOLAS formulas.

- Survey intervals and condition: annual/intermediate/periodical/renewal survey cycles that quantify the ship's continued fitness.

- Carriage requirements: number, type and placement of life-saving and fire appliances per person on board and ship length/trade.

## Required Data

The following data elements must be captured to support reporting and verification:

- Approved plans (general arrangement, fire control, lifesaving)   •   Stability booklet and loading-computer output   •   Survey and certificate status   •   Deficiency and detention records (PSC)   •   Maintenance records for safety equipment   •   GMDSS and navigation equipment records

## Reporting Frequency

Continuous certification: harmonised system of surveys (HSSC) with annual, intermediate, periodical and renewal surveys across the relevant certificates (typically a five-year cycle with intermediate at 2–3 years); port-State inspection is risk-based and can occur any call.

## Common Compliance Mistakes

- Letting certificates expire or surveys fall outside their window, triggering port-State detention.

- Operating the loading computer without a current, approved version, or deviating from approved loading.

- Carrying out equipment or structural changes without re-approval, invalidating the certificate basis.

- Under-maintaining life-saving and fire appliances, evidenced at drill or inspection as inoperative equipment.

- Overlooking the ISM/SOLAS interface, where an SMS failure can cascade into a SOLAS deficiency.

## How Software Can Automate Compliance

A purpose-built compliance platform materially reduces manual effort and error exposure in the following ways:

- A unified certificate and survey calendar across construction, equipment, radio and SOLAS-linked instruments, with escalating expiry alerts.

- Approved-plan vaulting with version control so only the current fire-control/lifesaving/stability documents are referenced.

- Loading-computer data capture that flags operations outside approved envelopes.

- PSC-deficiency tracking with corrective-action workflow and trend analysis to target high-risk equipment.

- Maintenance integration for life-saving and fire appliances so inoperative equipment is caught before inspection.

# 12. Cross-Regulation Analysis and Software Automation

Treated in isolation, each of the preceding instruments looks manageable. Treated together — as a fleet operator must — they reveal a single, shared data layer and a set of overlapping obligations whose efficient handling is the main determinant of compliance cost. This chapter consolidates those overlaps and sets out the architecture of a compliance platform that addresses all ten instruments at once.

## 12.1 The Shared Data Layer

Five data streams underpin almost every instrument: bunker delivery and consumption, voyage and distance, cargo and capacity, equipment and certificate status, and management-system records (audits, drills, non-conformities). Capturing these once, in a verified form, and re-using them across instruments eliminates the duplicated effort and inconsistency that produce most findings.

Table 2: Mapping of shared data to the instruments that consume it

| Data stream | Primary consumer instruments |
| --- | --- |
| Fuel consumption (by type) | IMO DCS, EU MRV, EU ETS, FuelEU, CII, MARPOL VI |
| Voyage / distance / hours underway | IMO DCS, EU MRV, EU ETS, FuelEU, CII |
| Cargo / capacity / transport work | EU MRV, CII, EEXI |
| Emission factors (TtW & WtW) | EU MRV, EU ETS, FuelEU, CII |
| Equipment & certificate status | SOLAS, MARPOL, EEXI (IEE), ISM (Doc/SMC), ISPS (ISSC) |
| Audits, drills, non-conformities | ISM, ISPS (and evidence base for all) |

## 12.2 Overlapping Obligations to Reconcile

- Tank-to-wake versus well-to-wake: EU MRV/ETS/CII are predominantly TtW; FuelEU is WtW. A platform must hold both factor sets and apply the right one per instrument.

- Geographic allocation: the 100%/50% intra-versus-extra-EU rule is common to EU MRV, EU ETS and FuelEU and should be computed once.

- Reporting calendar: EU MRV (30 April), FuelEU (30 June) and EU ETS surrender (September) form a tight six-month sequence that benefits from a single planning calendar.

- Management-system links: ISM and ISPS supply the evidentiary discipline that verifiers and port-State officers expect; environmental instruments should feed findings back into the SMS.

## 12.3 What a Compliance Platform Should Do

- Capture once, report many: ingestion of MFM, sounding, BDN, AIS/ECDIS, and record-book data into a single verified ledger that fans out to every instrument's required dataset.

- Calculation engines: centrally maintained emission factors, CII band edges, EEXI/EEDI reference lines, FuelEU limit trajectory and EU ETS phase-in percentages, so regulatory changes propagate without per-ship rework.

- Continuous projection: in-year CII rating, EU ETS surrender exposure and FuelEU compliance balance shown live, not computed only at year-end.

- Document vaulting: versioned Monitoring Plans, SEEMP (I–III), SMS/SSP, EEXI files and certificates with expiry and approval alerts.

- Verifier- and PSC-ready evidence packs: on-demand assembly of the data and documents a verifier or port-State officer will ask for, with full traceability.

- Auditability: immutable change logs that demonstrate how each reported figure was derived — increasingly important as enforcement tightens.

# 13. Conclusions and Recommendations

The ten instruments reviewed here are not a loose collection of rules but an increasingly coherent regime with a common data core. SOLAS, MARPOL, the ISM Code and the ISPS Code establish the safety and environmental floor that every ship must clear; IMO DCS, CII and EEXI add a global operational and technical efficiency layer; and EU MRV, EU ETS and FuelEU Maritime impose Europe's market-based and technology-forcing obligations on top. A company that treats these as separate workstreams pays for the same fuel and voyage data many times, and is exposed where the workstreams diverge.

The defensible recommendation is to consolidate around a single, verified data backbone — capturing consumption, voyage, cargo and equipment data once — and to let calculation engines and document vaults fan that data out to each instrument's requirements. The same platform should provide continuous, in-year projection of the metrics that carry real financial risk (CII rating, EU ETS surrender, FuelEU compliance balance), so that corrective action happens while there is still time for it to matter.

Finally, the compliance function should be integrated, not siloed. Environmental reporting, safety management and security operate on the same ships and crews; routing findings from one into the others — for example, surfacing a FuelEU shortfall in the next ISM management review — is the step that turns compliance from a cost into a management capability.